# Tableau Server Upgrade

# Run Setup

Follow these steps to upgrade a single-node installation of Tableau Server version 2018.2 or later.

Log on as a user with sudo access to the computer you are upgrading.

Navigate to the directory where you copied the .rpm or .deb Tableau Server package.

Use the package manager to install the Tableau Server package.

You must install the new version to the same location as the existing version. The install location must be the same on all nodes. Do not install to a location using a symbolic link or to a directory on a Network File System (NFS) volume.

On RHEL-like distributions, including CentOS, you have the option to install Tableau to a non-default location.

Default location—To install to the default location (/opt/tableau/tableau_server), run the following commands, where <version> is formatted as major-minor-maintenance (ex: 2019-2-5):

sudo yum update

sudo yum install tableau-server-<version>.x86_64.rpm

Non-default location—To install to a non-default location, you must use rpm -i. You will also need to install all dependent packages. See the note below.

Run the following command:

sudo rpm -i –-prefix /preferred/install/path tableau-server.rpm

# Run the upgrade script.

If Tableau Server is not stopped when you run the script, the script will let you know, and will offer to stop the server. You can also choose to stop the server before running the script using the tsm stop command. Tableau Server must be stopped to complete the upgrade.

The options you need to include depend on the version you are upgrading to:

Version 2019.3 or later:

sudo /opt/tableau/tableau_sever/packages/scripts.<version_code>/upgrade-tsm --accepteula